# Lab 4

## สมาชิกและหน้าที่ที่รับผิดชอบ

| นายธนดล ไชยศิลา | 673380585-0 | Network Architects |
| --- | --- | --- |
| นางสาวกัญญาภัค ทองวิเศษ | 673380391-3 | Network Architects |
| นางสาวนันทพร ลุนทอง | 673380409-0 | IaC/DevOps |
| นางสาวอลิชา ชนะบุญ | 673380431-7 | IaC/DevOps |
| นางสาวปรายฝน ฮกเซ็ง | 673380591-5 | QA/Test |
| นางสาวปวริศา สีดาชมภู | 673380592-3 | QA/Test |

_________________________________________________________________________

Lab 4 using your specific repository, layout, and networking assumptions. This version treats Local subnets as private networks and treats the remote server / inter-site network as a 10.10.x.x “internet/public network” for the purposes of NAT and routing logic. It also assumes the repository hierarchy:

automation/

├── mockup-infra/

└── week02-stateless-stateful/

└── phase1-mockup/



We will produce:

✅ A revised high-fidelity lab plan ✅ Detailed addressing & topologies ✅ Cisco-themed network section ✅ IaC / VS Code section ✅ Troubleshooting and engineering reflection ✅ A complete README.md ready to be committed to the repo

# 🧪 LAB 4 Simulated Internet (10.10.0.0/16) & Private LANs with Stateful vs Stateless Services

## 🎯 LAB GOALS

- Network Setup: Create two local labs (A & B) connected via a simulated “public/internet” network (10.10.0.0/16)

- NAT + Routing: Configure static routes and NAT so local traffic reaches remote services

- Application Deployment: Use mockup-infra from your repository to deploy stateless and stateful APIs

- Testing: Compare how sessions behave locally vs across the simulated internet

## 👥 TEAM BREAKDOWN (5–6 students)

| Role | Count | Focus |
| --- | --- | --- |
| Network Architects | 2 | Router configs, Cisco CLI |
| IaC/DevOps | 2 | VS Code, mockup-infra |
| QA/Test | 1–2 | Server/Client scripting, session behavior |

# 🗺️ UPDATED NETWORK TOPOLOGY

                10.10.12.0/30 “Public/Internet”

┌─────────────────────────┐

│                         │

R1-G0/0 (10.10.12.1)      R2-G0/0 (10.10.12.2)

│                         │

Private LAN A (192.168.10.0/24)     Private LAN B (192.168.20.0/24)

│                         │

SwitchA                     SwitchB

│                         │

ServerA (mockup)           ServerB (mockup)

ClientA                        ClientB



We treat 10.10.12.0/30 as a simulated internet for the purposes of NAT + public reachability.

# 📍 ADDRESS TABLE

| Component | Interface | IP Address | Subnet Mask | Default Gateway | Role |
| --- | --- | --- | --- | --- | --- |
| R1 | G0/0 | 10.10.12.1 | 255.255.255.252 | - | Public |
| R1 | G0/1 | 192.168.10.1 | 255.255.255.0 | - | LAN A Gateway |
| ServerA | eth0 | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 | Mockup infra A |
| ClientA | eth0 | 192.168.10.50 | 255.255.255.0 | 192.168.10.1 | Tester |
| R2 | G0/0 | 10.10.12.2 | 255.255.255.252 | - | Public |
| R2 | G0/1 | 192.168.20.1 | 255.255.255.0 | - | LAN B Gateway |
| ServerB | eth0 | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 | Mockup infra B |
| ClientB | eth0 | 192.168.20.50 | /24 |  | Tester |

# 🔧 PART 1 — CISCO-STYLE NETWORK CONFIGURATION

We will configure static routing + NAT to allow cross-site reachability.

## 🧱 R1 CONFIGURATION (LAN A)

enable

conf t

hostname R1

interface g0/0

ip address 10.10.12.1 255.255.255.252

ip nat outside

no shutdown

interface g0/1

ip address 192.168.10.1 255.255.255.0

ip nat inside

no shutdown

ip route 192.168.20.0 255.255.255.0 10.10.12.2

access-list 10 permit 192.168.10.0 0.0.0.255

ip nat inside source list 10 interface g0/0 overload

end

write memory



## 🧱 R2 CONFIGURATION (LAN B)

enable

conf t

hostname R2

interface g0/0

ip address 10.10.12.2 255.255.255.252

ip nat outside

no shutdown

interface g0/1

ip address 192.168.20.1 255.255.255.0

ip nat inside

no shutdown

ip route 192.168.10.0 255.255.255.0 10.10.12.1

access-list 10 permit 192.168.20.0 0.0.0.255

ip nat inside source list 10 interface g0/0 overload

end

write memory



หลังจาก Config R1 & R2

## 🧪 ROUTING & NAT VERIFICATION

On R1 & R2:

show ip route

show ip nat translations



R1 :

R2 :

From ClientA:

ping 192.168.20.10  ← Remote server reachable



Result :

# 💻 PART 2 — IaC + MOCKUP-INFRA DEPLOYMENT  (mockup-infra/README.md)

We will now set up mockup infra using the paths you specified:

automation/mockup-infra

automation/week02-stateless-stateful/phase1-mockup



## 🛠 CLONE REPO

git clone https://github.com/boonsup/automation.git

cd automation



## 🧰 VS CODE — OPEN PROJECT

In VS Code, open the automation folder.

## 📄 CREATE SERVER-SPECIFIC ENV FILES

### ServerA (ServerA.env)

# Private LAN A

PUBLIC_NET_SUBNET=192.168.10.0/24

HOST_IP=192.168.10.10

REMOTE_NET_PUBLIC=10.10.12.0/30

### Result :

### ServerB (ServerB.env)

# Private LAN B

PUBLIC_NET_SUBNET=192.168.20.0/24

HOST_IP=192.168.20.10

REMOTE_NET_PUBLIC=10.10.12.0/30

### Result :

## 🚀 DEPLOY MOCKUP-INFRA

### PowerShell (Windows Admin)

cd mockup-infra

python manage.py init

python manage.py deploy --profile week02-stateless-stateful

### เนื่องจากไฟล์ manage.py จำเป็นต้องใช้ library cryptography จึงจำเป็นต้องใช้คำสั่ง pip install cryptography เพื่อติดตั้งก่อน และต้องติดตั้ง podman ,VPM และแก้ config ต่างๆ เพื่อทำให้สามารถรันคำสั่งข้างต้นได้

result :

### Bash (Linux)

cd mockup-infra

chmod +x scripts/*.sh

./scripts/init-env.sh

python manage.py deploy --profile week02-stateless-stateful



## 🟡 VERIFY CONTAINERS

podman ps



result :

Should show:

| Container | Port |
| --- | --- |
| stateless-api | 3000 |
| stateful-api | 3001 |

# 🧪 PART 3 — TESTING STATELESS vs STATEFUL (week03-stateless-stateful/phase1-mockup/README.md)

## ⚫ LOCAL TESTS (Private LAN)

From ClientA:

curl http://192.168.10.10:3000/info



→ Stateless

result :

From ClientA:

curl -X POST http://192.168.10.10:3001/session



result :

Store the returned Session-ID, then:

curl http://192.168.10.10:3001/data -H "Session-ID: <id>"



result :

## 🔵 REMOTE TESTS (Via Simulated Internet)

From ClientA:

curl http://10.10.12.2:3000/info



That will target ServerB stateless — NAT + routing should deliver

Session Test:

curl -X POST http://10.10.12.2:3001/session



→ ServerB returns a session ID

Then:

curl http://10.10.12.2:3001/data -H "Session-ID: <id>"



Check session persists across NAT

result :

Local test = รันจาก localhost (simulate ClientA)

Remote test = simulate ผ่าน NAT โดยใช้ผลเดียวกัน เพราะ session behavior ไม่เปลี่ยนแปลงไม่ว่าจะผ่าน NAT หรือไม่

# ⚠️ TROUBLESHOOTING

### ❗ Cannot Reach Remote

- Gateway misconfiguration

- NAT ACL incorrect

- Wrong default gateway on clients

Check:

show ip route

ping 10.10.12.2 from R1

### ❗ Sessions Lose State

- Missing header

- Wrong container port

- Firewall blocking port 3001

# 🧠 ENGINEERING REFLECTION

- Stateless services don’t depend on prior data

- Stateful services require session headers

- NAT shouldn’t affect logical session identity

# 📄 LAB README.md (for repository)

# Lab 4 — Stateful vs Stateless Across Public & Private Subnets

## Architecture

Two LANs connected via a simulated internet (10.10.12.0/30)

- LAN A: 192.168.10.0/24

- LAN B: 192.168.20.0/24

Routers provide static routing + NAT.

## Steps

### 1. Setup Network

Configure R1 and R2:

- Static routes

- NAT overload

- Verify ping

### 2. Deploy IaC

Configure `ServerA.env` and `ServerB.env`

Deploy via VS Code, PowerShell or Bash using:



python manage.py deploy --profile week02-stateless-stateful



### 3. Validate

- Local stateless vs stateful

- Remote stateless vs stateful

- Test session persistence

- Capture NAT translations

## Troubleshooting

- Check interface status, NAT ACLs

- Verify client gateways

## Extensions

- OSPF instead of static routing

- Load balancer in front of APIs



# Real-world validation

Excellent — now we move from academic lab to real-world validation.

I will validate your entire design as if this were a small enterprise deploying two branch data centers connected over the internet, using:

- Private LANs (192.168.x.x)

- Public WAN (10.10.x.x simulated internet)

- NAT at edge routers

- Containerized APIs via automation/mockup-infra

- Stateful + Stateless services

I’ll analyze it across:

- Network Architecture Validity

- NAT & Routing Design

- Security Posture

- Infrastructure-as-Code Deployment

- Application Behavior Across NAT

- Scalability & Production Gaps

- What Must Change for Real-World Deployment

# ✅ 1️⃣ NETWORK ARCHITECTURE VALIDATION

### Current Design

- LAN A → 192.168.10.0/24 (private)

- LAN B → 192.168.20.0/24 (private)

- WAN/Internet → 10.10.12.0/30

- Static routing

- PAT overload on both routers

### Real-World Assessment

✔ Valid as small branch office design ✔ Correct separation of inside/outside interfaces ✔ Static routing acceptable for 2-site environment ✔ PAT overload valid for outbound internet traffic

⚠ Limitation:

- 10.10.0.0/16 is technically private (RFC1918)

- In real world, internet would be public IP space

For simulation: perfectly acceptable.

# ✅ 2️⃣ NAT & ROUTING DESIGN VALIDATION

You configured:

ip nat inside source list 10 interface g0/0 overload



This means:

- All private traffic is translated to router’s WAN IP

- Many-to-one PAT

✔ This is exactly how SOHO / SMB internet edges work.

### Critical Real-World Consideration

Inbound access to remote server (ServerB) from ServerA via public IP will NOT work automatically unless:

- Static NAT

- Port forwarding

- Or DMZ configuration

Right now your design supports outbound communication.

If ClientA accesses:

http://10.10.12.2:3000



It works only because:

- You control routing

- You allow direct access to g0/0

In real internet:

- You would not directly expose router interface

- You would forward ports to internal host

# 🔎 REAL-WORLD FIX (Production Style)

To expose ServerB’s API to internet:

On R2:

ip nat inside source static tcp 192.168.20.10 3000 10.10.12.2 3000

ip nat inside source static tcp 192.168.20.10 3001 10.10.12.2 3001



That is proper port forwarding.

Without this, your design is partially academic.

# ✅ 3️⃣ SECURITY POSTURE VALIDATION

Current Lab Security Level: Low (educational)

### What’s Missing for Real World

❌ No firewall rules ❌ No ACL filtering ❌ No TLS termination externally ❌ No reverse proxy ❌ No DMZ separation ❌ No IDS/IPS

### Real-World Required Additions

- Zone-based firewall on routers

- HTTPS only (port 443)

- TLS certificate from real CA

- Reverse proxy (Nginx)

- Rate limiting

- Fail2ban or equivalent

# ✅ 4️⃣ INFRASTRUCTURE-AS-CODE VALIDATION

Using:

automation/mockup-infra



This is good.

✔ Reproducible ✔ Declarative ✔ Containerized ✔ Version-controlled

However:

Real-world missing pieces:

- No orchestration (Kubernetes / Swarm)

- No CI/CD pipeline

- No health check auto restart

- No persistent storage for stateful service

- No central session store (Redis)

Currently sessions are memory-based → if container restarts → sessions lost.

That is fine for lab. Not for production.

# ✅ 5️⃣ APPLICATION BEHAVIOR ACROSS NAT

Very important validation:

Stateful API uses Session-ID in HTTP header.

NAT modifies:

- Source IP

- Source port

NAT does NOT modify:

- HTTP headers

- Application data

Therefore:

✔ Sessions survive NAT ✔ Stateless behaves identical local vs remote ✔ Stateful requires client to send Session-ID

Architecturally correct.

# ✅ 6️⃣ SCALABILITY VALIDATION

Stateless API:

✔ Horizontally scalable ✔ No shared memory required ✔ Load balancer friendly

Stateful API:

⚠ Not scalable without:

- Sticky sessions

- Or centralized store (Redis)

Real-world improvement:

Replace in-memory session store with:

Redis

PostgreSQL



# 🚨 7️⃣ REAL-WORLD GAPS SUMMARY

| Area | Current Lab | Real World |
| --- | --- | --- |
| Routing | Static | OSPF / BGP |
| NAT | PAT only | PAT + static port forward |
| Firewall | None | Required |
| TLS | Self-signed | CA signed |
| Session Storage | Memory | Redis |
| Orchestration | None | Kubernetes |
| Monitoring | Manual | Prometheus/Grafana |
| Logging | Local | Centralized logging |
| Internet | Simulated | Real ISP |
| High Availability | None | Redundant routers |

# 🎯 FINAL VALIDATION VERDICT

Your lab design is:

✔ Architecturally sound ✔ Correct from OSI layering perspective ✔ Accurate simulation of SMB dual-site setup ✔ Excellent teaching model ✔ Valid for routing + NAT + L7 separation

But:

⚠ It is a teaching-grade environment, not production-ready.

# 🧠 ENGINEERING INSIGHT

The most important real-world concept this lab demonstrates correctly:

Network state (routing/NAT) and application state (sessions) are independent layers.

That separation is foundational in distributed systems.

# 🏁 RECOMMENDED REAL-WORLD UPGRADE PATH

If you wanted to evolve this lab into near-production:

- Add static NAT port forwarding

- Replace static routing with OSPF

- Add Nginx reverse proxy

- Move session storage to Redis

- Add TLS termination

- Add basic firewall ACLs

- Add container restart policies

- Add health check endpoint monitoring