# LAB 6 — Resilient Internet + Secure Microservices (Optional)

Member

| นายธนดล ไชยศิลา | 673380585-0 |
| --- | --- |
| นางสาวกัญญาภัค ทองวิเศษ | 673380391-3 |
| นางสาวนันทพร ลุนทอง | 673380409-0 |
| นางสาวอลิชา ชนะบุญ | 673380431-7 |
| นางสาวปรายฝน ฮกเซ็ง | 673380591-5 |
| นางสาวปวริศา สีดาชมภู | 673380592-3 |

# LAB 6 Topology Evolution

Same physical layout as Lab 5:

Internet

|

R1 G0/0 (NAT Edge)

|

R1 S0/0/0 ---- R2 S0/0/0   (100.10.10.0/30)

|                     |

LAN A                LAN B



But now we introduce:

- OSPF Area 0 between R1 and R2

- IP SLA tracking default route

- ACL filtering WAN

- Controlled microservice exposure

ผลลัพธ์ :

# PHASE 1 — Replace Static with OSPF

## Why?

Static routes:

- No failover intelligence

- No topology awareness

- No scalability

OSPF provides:

- Automatic route exchange

- Convergence

- Metric-based path selection

## 🔹 R1 OSPF

router ospf 1

router-id 1.1.1.1

network 100.10.10.0 0.0.0.3 area 0

network 192.168.10.0 0.0.0.255 area 0

passive-interface g0/1



ผลลัพธ์ :

## 🔹 R2 OSPF

router ospf 1

router-id 2.2.2.2

network 100.10.10.0 0.0.0.3 area 0

network 192.168.20.0 0.0.0.255 area 0

passive-interface g0/1



ผลลัพธ์ :

## 🔹 Default Route Injection from R1

router ospf 1

default-information originate



Now:

- R2 automatically learns default route

- No static route needed

ผลลัพธ์ :

# PHASE 2 — WAN Failure Detection (IP SLA)

We simulate real ISP outage detection.

## On R1

ip sla 1

icmp-echo 8.8.8.8 source-interface g0/0

frequency 5

exit

ip sla schedule 1 life forever start-time now

track 1 ip sla 1 reachability

ip route 0.0.0.0 0.0.0.0 g0/0 track 1



If Internet fails:

- Default route removed

- OSPF stops advertising it

- R2 loses default cleanly

This mimics enterprise edge behavior.

ผลลัพธ์ :

เนื่องจากใน Cisco Packet Tracer ไม่รองรับการทดสอบ IP SLA จึงขออนุญาติข้ามขั้นตอนดังกล่าว

# PHASE 3 — Basic WAN Security

Currently: WAN is fully open.

We harden it.

## Block unsolicited WAN inbound

On R1:

ip access-list extended WAN-IN

permit tcp 192.168.20.0 0.0.0.255 192.168.10.0 0.0.0.255

deny ip any any log

interface s0/0/0

ip access-group WAN-IN in



Now:

- Only LAN B → LAN A allowed

- All other WAN inbound blocked

- Logged for visibility

ผลลัพธ์ :

# PHASE 4 — Microservice Exposure Control

Instead of exposing entire server:

Expose only gateway service.

Example:

ip nat inside source static tcp 192.168.10.10 8000 interface g0/0 8000



Now:

- Only port 8000 reachable from Internet

- Backend services remain private

This models:

- API Gateway pattern

- Real production practice

ผลลัพธ์ :

# PHASE 5 — Observability Foundation

Add:

### On Routers:

service timestamps log datetime msec

logging buffered 4096



ผลลัพธ์ :

### Enable NetFlow (optional advanced)

ip flow-export version 9



# PHASE 6 — Microservice Network Validation

Now validate:

### Test 1

ClientA → ServerB via OSPF path

### Test 2

R2 loses default if ISP unreachable

### Test 3

Internet cannot access backend ports

### Test 4

Gateway reachable from Internet

# LAB 6 Test Plan

| Test | Expected Result |
| --- | --- |
| OSPF neighbor | FULL |
| show ip route ospf | Remote LAN visible |
| ISP unplug | Default route disappears |
| Internet → port 8000 | Success |
| Internet → port 9000 | Blocked |
| LAN A ↔ LAN B | Success |

Test 1 — Ping LAN A ↔ LAN B :

Test 2 — Internet-PC ping หา ServerA :

Test 3 — ทดสอบ NAT Port 8000 :

Test 4 — ทดสอบ port 9000 ถูกบล็อก

# Readiness Score After LAB 6

| Category | Score |
| --- | --- |
| Routing | 9/10 |
| WAN Intelligence | 9/10 |
| Security | 6/10 |
| Microservice Exposure | 8/10 |
| Scalability | 8/10 |