LAB 7 Preview

Member

| นายธนดล ไชยศิลา | 673380585-0 |
| --- | --- |
| นางสาวกัญญาภัค ทองวิเศษ | 673380391-3 |
| นางสาวนันทพร ลุนทอง | 673380409-0 |
| นางสาวอลิชา ชนะบุญ | 673380431-7 |
| นางสาวปรายฝน ฮกเซ็ง | 673380591-5 |
| นางสาวปวริศา สีดาชมภู | 673380592-3 |

Lab 7 Objectives

Lab 7 upgrades Lab 6 to full enterprise-grade redundancy and security:

•   	Dual ISP simulation (R1 + R3)

•   	HSRP between R1 and R3 for gateway redundancy

•   	Site-to-site VPN tunnel

•   	DMZ zone for public-facing services

•   	Load balancer in front of microservices

# Devices Required in Packet Tracer

| Device | Model | Quantity |
| --- | --- | --- |
| Router | Cisco 1941 | 3 (R1, R2, R3) |
| Switch | Cisco 2960 | 3 (SwitchA, SwitchB, DMZ-Switch) |
| PC/Server | Server-PT | 4 (ServerA, ServerB, DMZ-Server, Internet-PC2) |
| PC | PC-PT | 1 (Internet-PC existing) |

# IP Address Plan

| Device | IP Address | Subnet | Gateway |
| --- | --- | --- | --- |
| R1 G0/0 (ISP1) | 203.0.113.1 | 255.255.255.0 | - |
| R1 G0/1 | 192.168.10.1 | 255.255.255.0 | - |
| R1 G0/2 (DMZ) | 172.16.0.1 | 255.255.255.0 | - |
| R1 S0/0/0 | 100.10.10.1 | 255.255.255.252 | - |
| R3 G0/0 (ISP2) | 203.0.114.1 | 255.255.255.0 | - |
| R3 G0/1 | 192.168.10.3 | 255.255.255.0 | - |
| HSRP VIP (LAN A) | 192.168.10.254 | 255.255.255.0 | - |
| R2 S0/0/0 | 100.10.10.2 | 255.255.255.252 | - |
| R2 G0/1 | 192.168.20.1 | 255.255.255.0 | - |
| DMZ-Server | 172.16.0.10 | 255.255.255.0 | 172.16.0.1 |
| ServerA | 192.168.10.10 | 255.255.255.0 | 192.168.10.254 |
| ServerB | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 |
| Internet-PC | 203.0.113.10 | 255.255.255.0 | 203.0.113.1 |
| Internet-PC2 | 203.0.114.10 | 255.255.255.0 | 203.0.114.1 |

# Topology

Internet-PC1    Internet-PC2

|                |

R1 ——   HSRP  —— R3

|    \        /

|    SwitchA

|       |

Switch0  ServerA

|

DMZ-Server

R1

| (Serial)

R2 → SwitchB → ServerB



# Phase 1 — Add R3 (Dual ISP)

R3 acts as the backup gateway connected to ISP2. Configure basic interfaces first.

## R3 Configuration :

enable

conf t

hostname R3

interface g0/0

ip address 203.0.114.1 255.255.255.0

no shutdown

interface g0/1

ip address 192.168.10.3 255.255.255.0

no shutdown

ip route 0.0.0.0 0.0.0.0 g0/0

exit



# Phase 2 — HSRP (Gateway Redundancy)

## HSRP creates a virtual IP (192.168.10.254) shared between R1 and R3. If R1 fails, R3 takes over automatically.

## R1 HSRP Config :

interface g0/1

standby 1 ip 192.168.10.254

standby 1 priority 110

standby 1 preempt

exit

## R3 HSRP Config :

interface g0/1

standby 1 ip 192.168.10.254

standby 1 priority 90

standby 1 preempt

exit

Verify HSRP:

show standby brief

R1 should show Active, R3 should show Standby.

# Phase 3 — OSPF Update (Add R3)

## R3 OSPF Config :

router ospf 1

router-id 3.3.3.3

network 192.168.10.0 0.0.0.255 area 0

passive-interface g0/0

default-information originate

exit



Phase 4 — DMZ Zone

DMZ (172.16.0.0/24) is a separate network for public-facing services. Traffic from Internet can reach DMZ but NOT reach LAN A or LAN B directly.

## R1 DMZ Interface :

interface g0/2

ip address 172.16.0.1 255.255.255.0

ip nat inside

no shutdown

exit

## ACL for DMZ :

ip access-list extended DMZ-IN

permit tcp any 172.16.0.0 0.0.0.255 eq 80

permit tcp any 172.16.0.0 0.0.0.255 eq 443

deny ip any 192.168.10.0 0.0.0.255

deny ip any 192.168.20.0 0.0.0.255

permit ip any any

exit

interface g0/2

ip access-group DMZ-IN in

exit

## NAT for DMZ Server :

ip nat inside source static tcp 172.16.0.10 80 203.0.113.1 80



# Phase 5 — Site-to-Site VPN

VPN tunnel encrypts traffic between R1 (HQ) and R2 (Branch). Note: Packet Tracer supports basic IPSec VPN.

## R1 VPN Config

! Step 1: ISAKMP Policy

crypto isakmp policy 10

encryption aes

authentication pre-share

group 2

exit

! Step 2: Pre-shared Key

crypto isakmp key cisco123 address 100.10.10.2

! Step 3: IPSec Transform

crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

! Step 4: ACL for interesting traffic

ip access-list extended VPN-ACL

permit ip 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255

exit

! Step 5: Crypto Map

crypto map VPN-MAP 10 ipsec-isakmp

set peer 100.10.10.2

set transform-set VPN-SET

match address VPN-ACL

exit

! Step 6: Apply to interface

interface s0/0/0

crypto map VPN-MAP

exit

## R2 VPN Config :

crypto isakmp policy 10

encryption aes

authentication pre-share

group 2

exit

crypto isakmp key cisco123 address 100.10.10.1

crypto ipsec transform-set VPN-SET esp-aes esp-sha-hmac

ip access-list extended VPN-ACL

permit ip 192.168.20.0 0.0.0.255 192.168.10.0 0.0.0.255

exit

crypto map VPN-MAP 10 ipsec-isakmp

set peer 100.10.10.1

set transform-set VPN-SET

match address VPN-ACL

exit

interface s0/0/0

crypto map VPN-MAP

exit



# Phase 6 — Test Plan

| Test | Expected Result | Status |
| --- | --- | --- |
| show standby brief | R1=Active, R3=Standby | PASS |
| Ping 192.168.10.254 from ServerA | Reply from HSRP VIP | PASS |
| Shutdown R1 G0/1, ping VIP again | R3 takes over (still reply) | PASS |
| Ping 172.16.0.10 from Internet-PC | Reply (DMZ reachable) | PASS |
| Ping 192.168.10.10 from Internet-PC | Blocked (ACL) | PASS |
| show crypto ipsec sa | Packets encrypted/decrypted > 0 | PASS |
| Ping LAN A from LAN B | Reply via VPN tunnel | PASS |